6G Hybrid Beamforming + Channel + RHS(GP)
Deep-Learning Dataset & Project Memory

Project reference document — prepared from the current simulation pipeline

# 1. Project Objective

Develop a learning-based beamforming system for a future 6G wireless link in which transmitter-side hybrid beamforming is combined with a wireless channel and a reconfigurable/holographic surface (RHS). The current numerical pipeline uses Rayleigh fading, BS MMSE beamforming, RHS excitation generation, and a Gradient Projection (GP) optimization to maximize a sum-rate objective. The long-term goal is to replace the computationally expensive GP stage with a Deep Learning model while preserving or approximating its beamforming performance.

# 2. Current System / Signal Flow

Conceptual end-to-end flow:

6G Transmitter → Hybrid Beamforming (RF + digital) → BS/RHS channel → RHS excitation / metasurface response → user channel → received signal → achievable rate.

In the current code, the BS channel is initialized as an identity matrix and MMSE beamforming is applied. The RHS→user channel is generated as a Rayleigh fading tensor. The effective channel is then supplied to the RHS GP optimizer. The RHS excitation matrix E is generated deterministically from wavelength, element spacing, RHS dimensions, and number of feeds.

# 3. Current Code: Important Tensor Definitions

| Variable | Typical Shape | Meaning |
| --- | --- | --- |
| H_RHS | [B, Nr, Nt, L] | Rayleigh RHS→user/channel tensor |
| H_BS | [B, Nt, Nt] | BS-side channel; currently identity |
| W_MMSE | [B, Nt, Nt] | MMSE beamforming matrix |
| H_testD | [B, L, Nr, Nt] | Effective channel passed to GP |
| E | [B, Nt, RFno] | RHS excitation matrix |
| M | [B, Nt, RFno] | RHS-applied excitation/amplitude state |
| w | [B, Nr, L] | User/RF-chain combining weights generated by GP |
| Rate | scalar | Average sum capacity / sum rate |

# 4. Dataset Philosophy

The dataset should not consist of 5,000 copies of the same topology with only a random Rayleigh realization changed. For a useful 6G beamforming model, each sample should represent a distinct wireless operating condition. Uniqueness should come from a controlled combination of geometry, channel realization, SNR/power, user/RF configuration, and spatial or propagation parameters.

# 5. Recommended Dataset Sample Definition

One dataset entry should represent one complete end-to-end wireless scenario:

System configuration: Nx, Nt, Nr, number of users/RF chains L, number of RHS feeds RFno.

Carrier/wavelength: frequency or wavelength.

RHS geometry: element spacing de/deH, array dimensions, feed positions.

Transmitter configuration: number of RF chains, digital precoder, analog RF beamformer.

Channel condition: BS→RHS and RHS→user channel matrices.

Propagation parameters: path loss, shadowing, LOS/NLOS condition, Doppler/coherence condition where applicable.

Noise and power: noise variance, transmit power, SNR.

RHS excitation: E and optimized RHS state M.

Optimization labels: GP-computed w, M, achievable sum rate, convergence iterations and runtime.

# 6. Inputs and Labels for Deep Learning

Recommended first supervised-learning formulation: feed the neural network the wireless state and transmitter/RHS configuration, and train it to reproduce the optimized GP solution.

| Category | Candidate Input | Recommended Target |
| --- | --- | --- |
| Channel | Real/imaginary H_BS and H_RHS or effective H_testD | Optimized beamforming variables |
| Hybrid BF | Digital precoder + analog/RF beamformer | Refined transmitter/RHS beamforming |
| RHS | E, geometry, feed positions | M / RHS coefficients |
| Operating point | SNR, Pt, noise variance | w and/or M |
| Topology | Nx, Nt, Nr, L, RFno | Beamforming solution |
| Performance | Not normally an input | Sum rate as auxiliary target |

For a first model, predict the optimized RHS state M and receiver/combining weights w. The sum rate should be retained as a validation metric and can also be used as an auxiliary training objective.

# 7. Complex-Valued Data Representation

Do not discard the imaginary component. For each complex matrix X, store Re(X) and Im(X) as separate channels. For example, H with shape [Nr,Nt,L] becomes a real tensor with shape [2,Nr,Nt,L]. E and M should similarly be represented by real and imaginary parts.

Avoid independently normalizing real and imaginary components when phase information would be distorted. Prefer a consistent global or per-feature scale, and store the scale factors with the dataset.

# 8. Creating More Than 5,000 Unique Samples

Recommended starting target: 8,000–20,000 scenarios. Generate them through a deterministic seed schedule so every scenario can be reproduced.

| Parameter | Example Range | Purpose | Variation Type |
| --- | --- | --- | --- |
| Nx | 10, 12, 16, 20, 24, 28, 30, 32, 36, 40 | RHS size | Discrete |
| Nr | 2, 4, 8 | Receive antennas | Discrete |
| L / RF chains | 2, 4, 6, 8 | Multi-stream operation | Discrete |
| RFno | 1, 2, 4 | RHS feeds | Discrete |
| Frequency | e.g. 28–100 GHz | 6G/FR3/sub-THz studies | Continuous/discrete |
| SNR | -10 to 30 dB | Noise/power diversity | Continuous |
| Element spacing | 0.1λ–0.5λ | RHS geometry | Continuous |
| Path loss | scenario dependent | Link budget | Continuous |
| Shadowing | e.g. 0–8 dB | Large-scale fading | Random |
| LOS/NLOS | LOS or NLOS | Propagation diversity | Categorical |
| User positions | random spatial coordinates | Spatial diversity | Continuous |
| Channel seed | unique per sample | Small-scale fading | Unique |

# 9. Recommended Generation Strategy

Create a configuration sampler that selects Nx, Nr, L, RFno, frequency, spacing, SNR and propagation parameters.

Generate physically meaningful geometry from the selected Nx and element spacing.

Generate the BS→RHS channel using a configurable channel model. Keep the current identity H_BS only as a debugging baseline, not as the final 6G dataset.

Generate the RHS→user channel. Start with Rayleigh fading for baseline experiments, then add Rician/clustered/LOS/NLOS models.

Generate transmitter hybrid beamforming: analog RF precoder/combiner under constant-modulus constraints plus digital baseband precoding.

Construct the effective channel supplied to the RHS.

Generate E from wavelength, geometry and feed positions.

Run the corrected GP optimizer to obtain the reference RHS solution M and combining/beamforming weights w.

Calculate sum rate, per-user rate, SINR, power constraint violation, convergence iterations and GP runtime.

Save the complete sample plus a metadata record and random seed.

Reject or regenerate samples that are duplicates, numerically invalid, non-convergent or violate physical constraints.

# 10. Suggested 8,000-Sample Dataset Split

Use 70% training / 15% validation / 15% test as a practical starting point: 5,600 train + 1,200 validation + 1,200 test = 8,000 samples.

Do not split purely at random if the same geometry/configuration appears in all partitions. Use grouped splits so that some combinations of Nx, SNR, frequency, user placement, or channel seeds are held out. This tests generalization rather than memorization.

# 11. Dataset Storage Recommendation

For the first implementation, use HDF5 or Zarr for large complex tensors and a separate CSV/Parquet metadata table. A single sample should have a stable sample_id.

Suggested logical structure:

/samples/{id}/H_BS_real, H_BS_imag

/samples/{id}/H_RHS_real, H_RHS_imag

/samples/{id}/H_effective_real, H_effective_imag

/samples/{id}/E_real, E_imag

/samples/{id}/W_HB_real, W_HB_imag

/samples/{id}/M_target_real, M_target_imag

/samples/{id}/w_target_real, w_target_imag

/samples/{id}/rate

/samples/{id}/metadata

# 12. Metadata to Store for Every Sample

sample_id and random_seed

Nx, Nt, Nr, L, RFno

carrier_frequency / wavelength

deH, array dimensions and feed coordinates

SNR, transmit power, noise variance

channel model and LOS/NLOS state

path-loss and shadowing parameters

user coordinates / angles / distances

hybrid beamforming configuration

GP step size, convergence tolerance and maximum iterations

GP convergence status and number of iterations

sum rate and per-user rates

generation timestamp and code/version identifier

# 13. Critical Correction for the Current Code

The present code should be treated as a prototype/debugging implementation, not yet as the final dataset generator. In particular, the BS→RHS channel is currently an identity matrix, so the MMSE stage does not represent a realistic wireless BS link. Also, the GP gradient implementation has been modified repeatedly to resolve tensor-shape issues. Before generating thousands of labels, freeze and validate one mathematically consistent reference implementation.

A second important point is the dimension convention. H_RHS is generated as [batch, Nr, Nt, L], while H_testD is rearranged to [batch, L, Nr, Nt]. The GP function must use one convention consistently. Avoid arbitrary repeat/expand operations merely to make tensors broadcast; such operations can change the mathematical meaning of the gradient.

# 14. Reference Baseline Experiments Before Dataset Generation

Experiment A: current identity BS channel + Rayleigh RHS channel.

Experiment B: realistic BS→RHS Rayleigh channel + Rayleigh RHS→user channel.

Experiment C: Rician/LOS channel.

Experiment D: multi-user interference with L > 1.

Experiment E: different SNRs and transmit-power constraints.

Experiment F: different RHS sizes and feed configurations.

Experiment G: hybrid beamforming with constant-modulus RF precoder.

# 15. Deep-Learning Objective

The core DL task should be framed as learning the mapping:

Wireless State + Hybrid BF + RHS Geometry + SNR  →  Optimized RHS/Beamforming State

The GP solution is the teacher/reference label. The neural network is not required to reproduce the GP internal iterations; it only needs to predict a feasible high-performance beamforming solution in one forward pass.

# 16. Recommended Single Deep-Learning Direction

For the eventual model, use a Transformer-based complex-valued/spatial attention architecture with separate real/imaginary feature channels. A practical implementation can embed each antenna/RHS element as a token, apply attention across spatial elements/users, and use configuration embeddings for SNR, frequency, geometry and RF-chain count.

The model should have two outputs: (1) RHS control/beamforming parameters and (2) optional rate prediction. Train with supervised loss against GP labels plus a differentiable or simulation-based rate loss and feasibility penalties.

# 17. Training Loss Concept

Beamforming reconstruction loss: distance between predicted and GP reference M/w.

Rate loss: maximize achievable sum rate produced by the predicted beamformer.

Power/constant-modulus/phase feasibility penalty.

Optional per-user fairness loss to avoid optimizing only the strongest user.

# 18. 6G-Oriented Extensions

Sub-THz / FR3 frequency variation.

Near-field spherical-wave propagation for extremely large RHS/antenna arrays.

Wideband OFDM and frequency-selective channels.

Mobility and Doppler.

Blockage and dynamic LOS/NLOS transitions.

Multi-user scheduling and fairness.

Energy efficiency as a second optimization objective.

Hardware impairments: phase quantization, amplitude errors, mutual coupling and RF-chain limitations.

Multi-RHS / multi-surface scenarios.

# 19. Project Roadmap

| Phase | Work | Success Criterion |
| --- | --- | --- |
| Phase 1 | Fix and validate GP mathematically | One scenario; compare dimensions and rate manually |
| Phase 2 | Build deterministic dataset generator | 8,000+ unique samples with metadata |
| Phase 3 | Train supervised baseline | Predict GP beamforming |
| Phase 4 | Add rate/constraint loss | Improve physical feasibility |
| Phase 5 | Test unseen configurations | Nx/SNR/frequency/user-position generalization |
| Phase 6 | Compare GP vs DL | Rate, latency, memory, power, convergence |
| Phase 7 | 6G channel extensions | Wideband, near-field, mobility, LOS/NLOS |
| Phase 8 | Deployment | Compress/quantize model for edge/PHY hardware |

# 20. Key Principle to Preserve

The dataset must contain the complete relationship between channel state, hybrid transmitter beamforming, RHS geometry/excitation, operating conditions and the optimized beamforming solution. The most important evaluation is not merely prediction MSE. The final DL solution must be evaluated by achievable sum rate, per-user rate, constraint satisfaction, inference latency and robustness on channel/configurations that were not present during training.

# 21. Next Implementation Step

Before producing the 5K–20K dataset, implement a single-sample reference function such as generate_sample(config, seed) that returns {inputs, GP_targets, metrics, metadata}. Validate it for 10–20 samples, then parallelize it and generate the full dataset. This prevents a shape or mathematical error in GP from contaminating thousands of training labels.

Document status: Project memory/reference — suitable for continuing the dataset-generation and DL-model work.